Adriann Liceralde

Dr. Brian McPherson

Carbon Science Research at the University of Utah

CSR Arduino Collection Project

Last Updated February 20, 2022

Information for the MicroSD Card Module

Introduction

- The MicroSD Card module is an electronic device that stores data from a microcontroller into a microSD card.

- Any microcontroller can control this sensor. However, this guide and the associate codes use an Arduino Uno or an Arduino Mega to operate the device.

- Communicating with the device is performed via the SPI method.

- The module is compatible with any microSD card.

Important Notes

- Only insert/remove a microSD card when no power supply is connected. Doing so while power is active may result in errors or damage.

- This module has six pins that need to be connected to an Arduino microcontroller. If any pin is slightly disconnected, then the module will not operate. If a module is not working, the most likely reason is a loose pin.

- The SD card file must be safely closed before power is disconnected. A Toggle Switch can be implemented to do this, which will be explained in the Wiring section.

Links

- Product Search for Module:

https://www.amazon.com/s?k=micro+sd+card+module+arduino&crid=24MLFBXJ18ZYD&sprefix=micro+sd+card+module+arduin%2Caps%2C214&ref=nb_sb_noss_2

- Product Info for Level Converter:

https://www.sparkfun.com/products/12009

- Arduino Library:

https://www.arduino.cc/reference/en/libraries/sdfat-adafruit-fork/

- Arduino Code:

https://github.com/RiceAllDay22/EGI_Arduino_Collection/tree/main/SDCardModule

Wiring

- In the following sections, four different examples are shown:

  - Arduino Uno without Logic Converter

  - Arduino Uno with Logic Converter

  - Arduino Mega without Logic Converter

  - Arduino Mega with Logic Converter

- Each example contains an Arduino board, a MicroSD card module, and a Toggle Switch.

  - The Toggle Switch is used to close the SD file safely.

  - When the switch is positioned “left” towards the 5V line, then the file remains open.

  - When the switch is positioned “right” towards the GND line, then the file closes.

- Two of the examples use a Logic Converter module.

  - This converter ensures that the MOSI, SCK, and CS pins are operating at 3.3 V, as required by the datasheet.

  - Those three pins can still operate at 5.0 V; however, doing so may result in errors or damage to the module and/or the SD card.

  - Therefore, it is highly recommended that a Logic Converter is implemented when using the microSD card module.

- The operating procedure, which applies to all the examples, is shown below:

  - 1. Assemble the circuit using one of the methods shown further in the guide

  - 2. Insert a microSD card into the slot

  - 3. Flip the switch to the “left” position

  - 4. Connect the power supply

  - 5. Upload the Arduino software

  - 6. The microSD card module boots up

  - 7. The circuit continuously stores data into the microSD card

  - 7. Flip the switch to the “right” position to close the file

  - 8. Disconnect the power supply

  - 9. Eject the microSD card from the slot

- The Arduino code is the same across all four examples.

  - The inclusion of the Logic Converter does not change the software.

  - No Arduino Mega-specific features are used in these examples. Therefore, the code is compatible with both the Uno and the Mega.

Wiring – Arduino Uno Method #1 (Simpler but NOT recommended)

- This example does not include the Logic Converter.

- If the application involves long-term use, then do not use this method because the chances for errors and damage are high.

Figure 1. Wiring and Schematic diagrams for the module via an Uno without a Logic Converter

Wiring – Arduino Uno Method #2 (Recommended for reliability)

- This example contains the Logic Converter, making the module more reliable and less likely to fail.

Figure 2. Wiring and Schematic diagrams for the module via an Uno with a Logic Converter

Wiring – Arduino Mega Method #1 (Simpler but NOT recommended)

- This example does not include the Logic Converter.

- If the application involves long-term use, then do not use this method because the chances for errors and damage are high.

Figure 3. Wiring and Schematic diagrams for the module via a Mega without a Logic Converter

Wiring – Arduino Uno Mega #2 (Recommended for reliability)

- This example contains the Logic Converter, making the module more reliable and less likely to fail.

Figure 4. Wiring and Schematic diagrams for the module via a Mega with a Logic Converter

Contact

For any questions or assistance, email Adriann Liceralde at adriann8399@gmail.com.